HC-05 Bluetooth Classic Baseline Replication Guide

ESP32 Dev Board Method for Configuring and Testing an HC-05, with Future Transition to the Real Impedance Microsystem

Purpose. This document captures the working procedure developed after the Arduino UNO R3 could not be used as the HC-05 programming/configuration bridge. It explains how to reproduce the working setup on another computer, including the laptop, and how the HC-05 baseline should later transfer to the real NBSS cell impedance device.

Important terminology. The HC-05 is not programmed with project code. It is configured with AT commands and then acts as a Bluetooth Classic serial cable. The application logic runs on a microcontroller, such as the ESP32 during testing or the ATmega328PB in the real device.

# 1. What Was Proven in This Chat

The working baseline path is:

ESP32 simulated data → UART → HC-05 → Bluetooth Classic → Windows COM port → PuTTY / app

The ESP32 dev board was used as a USB-to-UART bridge and later as a simulated data transmitter.

The HC-05 was placed into AT mode using KEY/EN tied to 3.3 V before power-on.

AT commands successfully returned responses such as OK, version, name, UART settings, and Bluetooth address.

Normal data mode was confirmed when Windows created Bluetooth COM ports and PuTTY on COM8 received serial data.

The final simulated-data test sent JSON packets once per second through the HC-05 to PuTTY.

# 2. Hardware and Software Used

| Item | Role |
| --- | --- |
| ESP32 dev board | Acts as USB-to-UART bridge for AT configuration and as simulated data transmitter. |
| HC-05 Bluetooth Classic module | Serial-over-Bluetooth module. Configured with AT commands; forwards UART bytes over Bluetooth. |
| Windows PC or laptop | Runs Arduino IDE, Device Manager, and PuTTY. In the first test, PC was used; this guide supports replication on the laptop. |
| Arduino IDE | Used to upload ESP32 sketches. |
| PuTTY | Used to open the HC-05 Bluetooth COM port and verify received data. |
| USB cable for ESP32 | Provides power and programming connection to the ESP32. No external power was used. |

# 3. Safety Rules Before Replicating on the Laptop

Use one power source only: laptop USB → ESP32 → HC-05.

Do not use the Arduino UNO R3 for this procedure unless its USB interface is confirmed healthy.

Do not use a DC barrel jack, MB102 breadboard power module, or second power supply while connected to the laptop.

Wire the HC-05 only while the ESP32 is unplugged.

The ESP32 uses 3.3 V GPIO logic, so no voltage divider is required between ESP32 TX and HC-05 RX.

The HC-05 breakout VCC was powered from ESP32 5V/VIN during the successful test. GND must be common.

# 4. Upload the ESP32 AT-Mode Bridge

Start with only the ESP32 connected to the computer. The HC-05 should not be wired yet.

Open Arduino IDE.

Select the ESP32 board. For a generic ESP32 dev board, ESP32 Dev Module worked.

Select the COM port that appears when the ESP32 is plugged in. In the successful PC setup, this was COM3.

Upload the following sketch. If upload stalls at “Connecting…”, hold BOOT until upload begins, then release.

/* ESP32_HC05_AT_Bridge.ino */
HardwareSerial HC05(2);

const int HC05_RX_PIN = 16; // ESP32 receives from HC-05 TXD
const int HC05_TX_PIN = 17; // ESP32 sends to HC-05 RXD

void setup() {
  Serial.begin(115200);
  delay(1000);

  // HC-05 full AT mode is commonly 38400 baud.
  HC05.begin(38400, SERIAL_8N1, HC05_RX_PIN, HC05_TX_PIN);

  Serial.println();
  Serial.println("ESP32 <-> HC-05 AT bridge ready.");
  Serial.println("Serial Monitor: 115200 baud, Both NL & CR.");
  Serial.println("Type AT and expect OK if HC-05 is in AT mode.");
}

void loop() {
  while (Serial.available()) {
    HC05.write(Serial.read());
  }
  while (HC05.available()) {
    Serial.write(HC05.read());
  }
}

After upload, open Serial Monitor at 115200 baud and Both NL & CR. Press EN/RESET if needed. You should see the bridge-ready message.

# 5. Wire the HC-05 to the ESP32

Unplug the ESP32 before wiring. Use the following connections:

| HC-05 Pin | ESP32 Pin |
| --- | --- |
| GND | GND |
| VCC | 5V / VIN |
| TXD | GPIO16 (ESP32 RX) |
| RXD | GPIO17 (ESP32 TX) |
| KEY/EN | 3.3V only for AT mode |

HC-05 TXD → ESP32 GPIO16
ESP32 GPIO17 → HC-05 RXD
HC-05 GND  → ESP32 GND
HC-05 VCC  → ESP32 5V/VIN
HC-05 KEY/EN → ESP32 3.3V for AT mode only

# 6. Enter AT Mode and Configure the HC-05

With KEY/EN connected to ESP32 3.3 V before power-on, plug the ESP32 into USB.

The HC-05 LED should blink slowly. This was the successful AT-mode indicator.

Open Serial Monitor at 115200 baud and Both NL & CR.

Type AT. The expected response is OK.

Useful commands used or checked during testing:

AT
AT+VERSION?
AT+NAME?
AT+NAME=HC05_A
AT+UART?
AT+UART=9600,0,0
AT+ROLE=0
AT+ADDR?

If AT gives no response: first verify the HC-05 is still slow blinking, then verify TX/RX wiring, then run the ESP32 loopback test by connecting GPIO17 directly to GPIO16. In the successful troubleshooting path, typing “hello” echoed back, proving the ESP32 bridge and pins were working.

# 7. Return to Normal HC-05 Data Mode

Unplug the ESP32.

Disconnect KEY/EN from 3.3 V.

Leave VCC, GND, TXD, and RXD connected.

Plug the ESP32 back in. The HC-05 should fast blink, meaning normal Bluetooth mode and waiting to pair/connect.

# 8. Upload the ESP32 Normal Bridge and Verify COM Ports

/* ESP32_HC05_Normal_Bridge.ino */
HardwareSerial HC05(2);

const int HC05_RX_PIN = 16;
const int HC05_TX_PIN = 17;

void setup() {
  Serial.begin(115200);
  delay(1000);
  HC05.begin(9600, SERIAL_8N1, HC05_RX_PIN, HC05_TX_PIN);

  Serial.println();
  Serial.println("ESP32 <-> HC-05 normal data bridge ready.");
  Serial.println("USB Serial Monitor: 115200 baud.");
  Serial.println("HC-05 UART: 9600 baud.");
}

void loop() {
  while (Serial.available()) {
    HC05.write(Serial.read());
  }
  while (HC05.available()) {
    Serial.write(HC05.read());
  }
}

Pair the HC-05 in Windows Bluetooth settings. Common PINs are 1234 or 0000.

Open Device Manager → Ports (COM & LPT). In the successful setup, COM3 was ESP32 USB, while COM7 and COM8 were Bluetooth serial ports.

Open Arduino Serial Monitor on COM3 at 115200 baud.

Open PuTTY on the Bluetooth COM port at 9600 baud. In the successful setup, COM8 was the correct HC-05 terminal.

Typing “hello” in PuTTY appeared in Arduino Serial Monitor, proving PuTTY → HC-05 → ESP32 worked.

# 9. Upload the Simulated Data Transmitter

After the normal bridge test worked, the ESP32 bridge was replaced with a transmitter sketch. This made the ESP32 generate JSON packets and send them through the HC-05 once per second.

/* ESP32_HC05_Simulated_Data.ino */
HardwareSerial HC05(2);

const int HC05_RX_PIN = 16;
const int HC05_TX_PIN = 17;
const unsigned long SEND_INTERVAL_MS = 1000;
const char* DEVICE_ID = "HC05_A";

const float R_SERIES = 10.0;
const float R_PARALLEL = 500.0;
const float CAPACITANCE = 5e-6;

const float frequencies[] = {
  100, 150, 220, 330, 470, 680, 1000,
  1500, 2200, 3300, 4700, 6800, 10000
};
const int NUM_FREQUENCIES = sizeof(frequencies) / sizeof(frequencies[0]);
int frequencyIndex = 0;
unsigned long lastSendTime = 0;

float randomFloat(float minValue, float maxValue) {
  return minValue + ((float)random(0, 10000) / 10000.0) * (maxValue - minValue);
}

String buildSimulatedPacket() {
  unsigned long timestamp = millis();
  float frequency = frequencies[frequencyIndex];
  frequencyIndex = (frequencyIndex + 1) % NUM_FREQUENCIES;

  float omega = 2.0 * PI * frequency;
  float rcTerm = omega * R_PARALLEL * CAPACITANCE;
  float denominator = 1.0 + (rcTerm * rcTerm);
  float realParallel = R_PARALLEL / denominator;
  float imagParallel = -(omega * CAPACITANCE * R_PARALLEL * R_PARALLEL) / denominator;
  float realImpedance = R_SERIES + realParallel + randomFloat(-0.5, 0.5);
  float imaginaryImpedance = imagParallel + randomFloat(-1.0, 1.0);
  float humidity = 52.0 + 2.0 * sin(timestamp / 60000.0) + randomFloat(-0.25, 0.25);

  String json = "{";
  json += "\"timestamp\":" + String(timestamp) + ",";
  json += "\"deviceId\":\"" + String(DEVICE_ID) + "\",";
  json += "\"frequency\":" + String(frequency, 1) + ",";
  json += "\"realImpedance\":" + String(realImpedance, 3) + ",";
  json += "\"imaginaryImpedance\":" + String(imaginaryImpedance, 3) + ",";
  json += "\"relativeHumidity\":" + String(humidity, 2);
  json += "}";
  return json;
}

void setup() {
  Serial.begin(115200);
  delay(1000);
  HC05.begin(9600, SERIAL_8N1, HC05_RX_PIN, HC05_TX_PIN);
  randomSeed(analogRead(34));

  Serial.println();
  Serial.println("ESP32 HC-05 simulated data transmitter started.");
  Serial.println("Sending one JSON packet per second at 9600 baud.");
}

void loop() {
  unsigned long now = millis();
  if (now - lastSendTime >= SEND_INTERVAL_MS) {
    lastSendTime = now;
    String packet = buildSimulatedPacket();
    HC05.println(packet);
    Serial.println(packet);
  }
}

Serial Monitor on COM3 showed JSON once per second.

PuTTY on COM8 at 9600 baud received the same JSON stream through Bluetooth Classic.

This completed the HC-05 baseline communication test.

# 10. Future Step: Replacing the ESP32 with the Real Device

Real device identified from the Hilton Head publication. The publication describes the real microsystem as an AD5933 analog front end controlled by an ATmega328PB microcontroller, with processed data transmitted wirelessly using an HC-05 Bluetooth module. The AD5933 performs impedance conversion/network analysis, the ATmega328PB handles control and data processing, and the web application records/visualizes frequency, real and imaginary components, magnitude, phase, and timestamps.

COTS IDE / impedance electrodes → AD5933 AFE → ATmega328PB → UART → HC-05 → Bluetooth Classic → app

The ESP32 does not need to remain in the final system. It was a temporary known-good UART data source. The final task is to make the ATmega328PB-based data collection board satisfy the same external interface contract.

| Interface Requirement | What the Real Device Must Do |
| --- | --- |
| UART settings | Transmit to HC-05 at 9600 baud, 8 data bits, no parity, 1 stop bit. |
| Electrical logic level | If ATmega328PB UART TX is 3.3 V, it can feed HC-05 RX directly. If it is 5 V, add a divider or level shifter before HC-05 RX. |
| Wiring | ATmega TX → HC-05 RXD; ATmega RX ← HC-05 TXD; common GND; stable HC-05 VCC. |
| Packet format | Send one complete newline-terminated JSON packet per sample. |
| Device identity | Use unique HC-05 Bluetooth names and unique JSON deviceId values for each node. |
| App compatibility | Keep the same external data format so the app does not care whether the source is ESP32 simulated data or the real ATmega/AD5933 board. |
| Sensor data | Replace simulated impedance/humidity with AD5933 real/imaginary data and real humidity measurements where applicable. |

# 11. Laptop Replication Checklist

Install Arduino IDE and ESP32 board support if not already installed.

Confirm the ESP32 appears as a COM port when plugged in.

Upload the AT bridge sketch with only ESP32 connected.

Wire HC-05 to ESP32 while unplugged.

Enter AT mode by connecting KEY/EN to 3.3 V before power-on.

Confirm slow blink and AT → OK.

Configure UART to 9600,0,0 and optionally rename the HC-05.

Remove KEY/EN from 3.3 V and confirm fast blinking normal mode.

Pair HC-05 in Windows Bluetooth settings.

Use Device Manager to identify ESP32 COM port and Bluetooth COM ports.

Use PuTTY at 9600 baud on the correct Bluetooth COM port.

Upload simulated data transmitter and confirm JSON packets arrive through PuTTY.

Only after successful replication, proceed to real sensor or real device integration.

# 12. Source Notes

Internal project conversation: ESP32 method developed after the UNO R3 USB interface was not recognized by Windows.

Hilton Head project publication: HH26_0126_FBK_v6.pdf, especially the Materials and Methods section describing AD5933, ATmega328PB, HC-05, power management, and web application data handling.